The files for these modules can be downloaded here:

https://ucsf.box.com/s/m903b5t6p9mhfsfs0at5nz25wvwszw5d

Module 1:  Introduction to ImageJ

- Opening an image in Fiji:  For List Items 2-4, we will use “Clone4.tif”  This is a 4-color z-stack of a Drosophila ovariole.

  - Metadata  With the image in the foreground, click “i" to see the metadata for the file

  - Channel tool and Brightness/Contrast tool:

    - Image  Adjust  Brightness/contrast

    - Image  Color  Channels tool

- Manipulating image

  - Brightness and contrast sliders

  - Merge vs color vs black and white

  - Lookup tables

  - Z-slices; intensity projections

- Exercise 1:  Converting images into panels of a figure

  - Record actions  Plugins   Macros  Record…  This allows you to keep track of what you did

  - Rotate image

    - Use the line tool to draw a line that is parallel to your final horizontal.  For the Clone4 image, this would be from the pointy part of the tissue in the upper right, down and to the left, passing through the middle of the mass of cells.

    - Analyze  measure to determine the angle of the line

    - Image  Transform  Rotate and then put the angle of the line in the box (be sure to make it a negative angle if the measure tool indicated that the angle of the line is negative)

  - Crop image

    - Decide on an aspect ratio (for these images, I like a 2 width to 1 height) and go to Edit  Select  Specify…

    - Set the numbers according to your aspect ratio to make a box big enough to encompass the part of the image you want to keep.  E.g. for Clone4, 512 x 256 works well

    - Click ok and then click inside the box to move it into position

    - Image  Crop

  - Save as tif  This is a lossless image format that preserves metadata and z-stacks.  You can return to it later to change channel colors, combinations, etc.

  - Save as png This is the image you will put into your presentation or figure.  Png is also a lossless file format, but it doesn’t preserve metadata or z-stacks so it is compatible with other image layout programs, like Preview, Acrobat, Powerpoint, Photoshop, and Inkscape.

  - Select different channels, zoomed-in insets, etc. and save them as a png file too

  - Copy text from the Macro Recorder to a text file, add a header with date and notes, and save to your Code folder.

- Exercise 2:  Image analysis:  File  Open Samples  Fluorescent Cells

We will walk through the steps to measure tubulin in the nucleus and actin at cell periphery

  - Record actions  Plugins   Macros  Record…  This allows you to keep track of what you did

  - Duplicate image  Image  Duplicate…  I duplicate the image at multiple steps so I don’t lose the previous image if I want to go back

  - Background subtraction

    - Use the Channels tool to view a single channel in monochrome (Greyscale).

    - Draw a square in a black area and click Cmd + m to measure the background intensity

    - Click somewhere else to get rid of the square and then subtract background:

      - Process  Math  Subtract…

    - A dialog box will pop up asking if you want to process all 3 images.  CLICK “NO”!

    - Repeat for the other two channels

  - Measurements

    - Defining which measurements to use:  Analyze  Set measurements…

    - ROI vs line:

To measure nuclear tubulin, draw ROI by hand around a nucleus in DAPI channel, switch to tubulin (green) channel and measure (cmd + m)

To measure actin at cell periphery, draw line using the freehand line or segmented line tool around the outside of the cell in the actin (red) channel and plot profile (Analyze  Plot Profile)

    - Saving data  When the Results window is in the foreground, the menu options at the top of the screen change.  With these menus, choose File  save as and save as into your Results folder

- Image Segmentation with thresholding only File  Open Samples  Fluorescent Cells  crop out words at the bottom

When you ask a program to automatically segment an image, what you’re really asking is for it to detect the boundaries of the objects (cells, tissues, etc.) of interest.  This is a binary process (inside/outside the boundary) so ImageJ uses a binary “mask” to perform this task.

  - Record actions  Plugins   Macros  Record…

  - Duplicate image

  - Set threshold*  Image  Adjust  Threshold…  Adjust slider, make sure “Dark Background” is checked, and click “Apply”

*Note:  Threshold must be applied to monochrome image

  - Create ROIs  Analyze  Analyze Particles…

First, keep size as 0–infinity; Display results and Add to Manager should be checked

Duplicate the image and redo with size as 50–infinity

  - ROIs will show up in the ROI Manager.  To apply them to the image, bring the original image to the foreground and then uncheck and recheck the Show All box in the ROI Manager window

  - Use ROIs to measure nuclear tubulin Change to the tubulin channel and click measure in the ROI manager

  - Save all images and measurements in your Results folder

  - Copy text from the Macro Recorder to a text file, add a header with date and notes, and save to your Code folder.

- Image segmentation with thresholds, watershed and voronoi:  File  Open Samples  Blobs

  - Record actions  Plugins   Macros  Record…

  - Duplicate image

  - Set threshold  Image  Adjust  Threshold…  Adjust slider, make sure “Dark Background” is unchecked, and click “Apply”

  - Use watershed to split touching objects:  Process  Binary  Watershed

  - Use Voronoi to find boundaries between objects:   Process  Binary  Watershed

  - Combine boundaries with original image:  Image  Color  Merge channels…

Module 2:  Trainable Weka Image Segmentation

      - Open  “Day1_control.jpg” from the Box folder (link above)

      - Open in Fiji

      - Record actions  Plugins   Macros  Record…

      - Open Weka Segmentation Window:  Plugins  Segmentation  Trainable Weka Segmentation

      - Use the magnifying glass to zoom in on a region of the image where there are a lot of eggs

      - Use the lasso tool to draw an outline around an egg and click “Add to class 1”  It will turn red

      - Repeat until you have outlined 10-15 eggs

      - Use the lasso tool to draw some outlines around the background.  Select regions in different places to capture the range of backgrounds (e.g. along the edge, near the edge, near the center).  Click “Add to class 2” after drawing each outline.  When you are done, it should look like the image in Figure 1.

Figure 2

Figure 1

      - Click “Train Classifier”

      - Click “Save Classifier” and save it in your Code folder

      - Click Create Result  What do you think of the result?  Compare to Figure 2, which was generated using a model that had much more training.

      - With the resulting image in the foreground, chose Image   Type  8-bit (even though 8-bit is already checked).  The image will turn into greyscale.

      - Make the image binary  Process   Binary  Make binary

      - Analyze particles Analyze   Analyze particles…

        - Select the check boxes for Display results, Add to Manager, and Include holes

      - Reopen the original image

      - Uncheck and then recheck the Show All box in the ROI Manager

      - Select the top ROI in the ROI manager, scroll down to the bottom ROI, hold down the shift key and then select the bottom ROI manager.  This will select all the ROIs in the ROI manager

      - Click the Measure button in the ROI manager

      - Save the results as a .csv file in your Results folder

      - Save all images in your Results folder

      - Copy text from the Macro Recorder to a text file, add a header with date and notes, and save to your Code folder.

      - Optional additional step:  Try adjusting the display values with the B&C slider to enhance the contrast between the eggs and the background and reapply the classifier as follows:

        - Save the file

        - Open the Weka Classifier

        - Click “Load classifier” and select the .model file you saved in step 9 above

        - Select the modified file you saved in Step 21a above

        - Click “No” when it asks you if you want to create a probability mask

        - Proceed with steps 11-19 above.

      - Consider what other parameters you could quantify to improve the segmentation.  For example, eggs have a very uniform area, so a filter that eliminates or flags objects above a threshold area might help.

[Optional] Module 3:  Cell Profiler

For this module, you will need to download Cell Profiler from here:  https://cellprofiler.org/releases

The built-in Cell Profiler tutorial is very good.  To follow it:

- Click the “Try Example” button on the Welcome Screen, and click “Ok”

- Close the Welcome Window.  In the window underneath, you will see the example files listed in the central window pane and the steps of the example pipeline in the pane on the left.

  - Double click on the 01_POS002_D.TIF file to open it and view it

  - Open a few others if you like.  Note that the files are 2D, single-channel images

- Click through the first four rows on in the panel on the left:

  - Images shows you which images are available to the program

  - Metadata gives you the option to extract metadata (if available) from your image files

  - NamesAndTypes is where you specify how the files in your list are related to each other.

    - Click “Update”.  You can see a table that shows that files with the same number following the POS label are part of the same image and files ending in the same letter before the .TIF ending are the same channel (e.g. D.TIFF are DAPI, etc.).  These rules are set using the dropdown menus above and can be modified for your files.

  - Groups gives you the opportunity to add additional tags (metadata) to your image (e.g. WT vs EXPT)

- The lines below the thin grey divider are the individual steps in the pipeline.  When creating your pipeline (or viewing someone else’s for the first time, as you are doing here) you will want to go through them one at a time.  Once you are happy with your pipeline, you can run the entire pipeline at once.

- To through the steps one at a time, click “Start Test Mode”

- Click the green triangle next to the first “Crop” line to run this step

  - Look at the settings for this step so you can understand what happened:  It started with the input image OrigBlue and saved an output image CropBlue that was cropped with a rectangle that was placed according to the indicated coordinates.

- So you can see how to automatically run through more than one step at a time, click the pause button in the IdentifyPrimaryObjects row (it will turn yellow) and then click Run

  - It will automatically do the next two Crop steps and then pause at the IdentifyPrimaryObjects step.

- Click on the green arrow in the IdentifyPrimaryObjects row to run that step

  - Look at the settings for this step so you can understand what happened.  It started with the input image CropBlue and is set to identify primary objects that the user is calling “Nuclei”.  The Cell Profiler creators recommend that you start by trying to segment nuclei if possible.  It will do this by looking for objects that match the given parameters about object diameter, thresholding settings, smoothing, etc.  Feel free to modify these parameters to see how it changes the segmentation accuracy if you want but put it back before proceeding.  A particularly interesting parameter to change is the Thresholding method.  Otsu, for example, doesn’t work well here but is good if your image has more than one thresholding boundary.

- Click on the green arrow in the IdentifySecondaryObjects row to run that step

  - Look at the settings for this step so you can understand what happened.  It took the CropGreen image (which has membrane signal) and the “Nuclei” objects identified in the previous step as inputs and follows a process using the parameters in this window to expand outward from the nuclei until it finds cell boundaries in the Green channel.

    - They seem happy about this result, but I’m not very impressed.  There is a 1:1 correspondence so it would be useful for counting cells, but the purple lines do not overlap with the cell membranes very well.

- Click on the green arrow in the IdentifyTertiaryObjects row to identify the cytoplasm.

- Click through each of the remaining rows to perform the various types of quantification.

  - You can’t write to a spreadsheet in Test mode; if you want to do that, you have to exit test mode and click start analysis